# Rationale Document: Adversarial Quiz -

# Logic & Pitfalls

Correct Answer Validation & Distractor Analysis

Adverserial Quiz Link: https://forms.gle/dgQuWyx6dfwErU4ZA

## Q1: Correct Answer: B

- Validation: The morphological gradient creates a ridge; as the structuring element B grows, the watershed line ("dam") thickens due to a wider transition area.

- Traps: (A): Regional minima ("sinks") represent homogeneous interiors or background, not boundaries, (C): Homogeneous regions form "catchment basins," while the watershed transform identifies the lines between them, (D): Hole-filling is a separate post-processing morphological operation, not part of the core topographic partitioning.

## Q2: Correct Answer: C

- Validation: The number of regions produced equals the number of regional minima. Beucher & Meyer use "homotopy modification" to force minima onto specific markers, while Bai et al. train CNNs to create "energy maps" where each object is a single basin.

- Trap: (A): Simple thresholding fails to account for topological basin structures and may merge objects or miss boundaries, (B): Excessive smoothing preserves boundaries poorly and often fails to remove irrelevant minima without destroying actual object contours, (D): Changing grid connectivity affects line thickness and implementation but does not solve the core issue of having too many regional minima.

## Q3: Correct Answer: B

- Validation: To ensure every point on an open surface (like a triangular mesh) is assigned to a basin, a "virtual pit" is placed at the boundary. This formal minimum captures downhill paths that would otherwise "fall off the edge," keeping the Pfaltz graph topographically stable.

- Trap: (A): The virtual pit is a minimum (lowest point) to capture flow; a maximum would act as a peak or ridge, (C): Virtual pits are specific tools for stabilizing open meshes or discrete elevation data, not a general requirement for all gradient images, (D): Curvature flow is a filtration/smoothing method, mathematically distinct from the topological definition of pits and watersheds.

## Q4: Correct Answer: A

- Validation: Flooding the light function (f) segments dark objects as basins. Flooding the gradient modulus (g) treats the rate of change as "height," where "ridges" (watersheds) mark steep transitions, enabling precise boundary detection regardless of object brightness.

- Trap: (B): Though the flooding mechanic is identical, the results differ: one segments by intensity (homogeneity), the other by gradient (transition), (C): In the gradient modulus, high values (peaks) are edges; homogeneous object interiors are "valleys" (minima), not peaks, (D): A key advantage is that the method is non-parametric, providing closed contours without needing arbitrary thresholds.

## Q5: Correct Answer: D

- Validation: The watershed transform treats images as topographic surfaces where local minima are "sinks" for catchment basins. Because gradient images contain numerous small minima from noise or texture, the algorithm assigns a region to every single one, resulting in a highly fragmented "over-segmented" image.

- Trap: (A): The region count is determined by the number of minima, not a fixed bit-depth (e.g., 256), (B): The transform partitions the entire image into basins and ridge lines regardless of original pixel brightness, (C): High-pass filters amplify noise and create too many irrelevant regions rather than merging objects.

## Q6: Correct Answer: C

- Validation: The marker-controlled approach eliminates irrelevant minima using morphological reconstruction (homotopy modification). All unmarked catchment basins are "filled" and turned into non-minima plateaus, ensuring regions are defined only by the markers.

- Trap: (A): Over-segmentation is resolved by mathematically imposing minima at marker locations, not merely by blurring noise or standard preprocessing, (B): This method specifically controls the total number of regions rather than simply adjusting edge contrast, (D): The modification is applied directly to the segmentation function (the topographic surface), not just used to create a binary mask.

## Q7: Correct Answer: B

- Validation: A model can show high average performance yet still fail unpredictably in 20–33% of cases. These catastrophic failures pose clinical risks, and mean metrics like Dice often hide them, compromising safety.

- Trap: (A) Dice can detect failures, but averaging dilutes severe errors, making models seem more reliable than they are, (C) CF Rate only measures how often failures occur; it doesn’t fix them, (D) Metrics like Dice/Jaccard do capture overlap issues, the problem is reliance on averages instead of worst-case reliability.

## Q8: Correct Answer: B

- Validation: Traditional watershed methods over-segment due to small gradient changes creating too many basins. DWT instead learns an energy landscape where each object forms a single basin, so a simple cut produces correct instances without fragmentation.

- Trap: (A) DWT avoids iterative methods like RNNs, keeping runtime constant regardless of object count, (C) The cut only extracts connected components; the learned energy map itself prevents over-segmentation, (D) Radial bin template-matching is used by other models; DWT keeps the watershed idea but improves it by learning the energy surface.

## Q9: Correct Answer: B

- Validation: Watershed first splits the image into small regions (basins), which become graph nodes instead of pixels. This reduces complexity and speeds up max-flow/min-cut, combining watershed’s boundary detection with graph cut to fix over-segmentation.

- Trap: (B) Manual input is still required (inner/source and outer/sink contours); watershed runs between them, (C) The method depends on minimizing an energy function (data + smoothness terms), (D) Graph cut is applied to the region between contours, not just the inner contour.

## Q10: Correct Answer: C

- Validation: Region-based active contours use inside/outside statistics, making them robust to noise. Level set theory enables contours to split, merge, and change shape automatically.

- Trap: (A) Edge-based methods rely on gradients and are noise-sensitive; Sobel can produce broken boundaries, (C) Classic snakes need good initialization near edges, while region-based models allow flexible starting positions, (D) Kernel filters often break boundaries; active contours maintain continuity and, with level sets, handle complex shapes.

# References

- Beucher, S. & Meyer, F. (1993). The Morphological Approach to Segmentation: The Watershed Transformation. Mathematical Morphology in Image Processing.

- Bai, M. & Urtasun, R. (2017). Deep Watershed Transform for Instance Segmentation. CVPR. (Foundational for the "Energy Map" concept).

- Baswaraj, D., Govardhan, A. & Premchand, P. (2012). Active Contours and Image Segmentation: The Current State of the Art. Global Journal of Computer Science and Technology.

- Beucher, S. & Lantuejoul, C. (1979). Use of Watersheds in Contour Detection. International Workshop on Image Processing, Real-Time Edge and Motion Detection/Estimation.

- Beucher, S. & Meyer, F. (1993). The Morphological Approach to Segmentation: The Watershed Transformation. Mathematical Morphology in Image Processing.

- Kaur, M. & Jindal, G. (2011). Medical Image Segmentation using Marker Controlled Watershed Transformation. IJCST.

- Lou, S., Pagani, L., Zeng, W., Jiang, X. & Scott, P.J. (2020). Watershed segmentation of topographical features on freeform surfaces and its application to additively manufactured surfaces. Precision Engineering.

- MathWorks (n.d.). Marker-Controlled Watershed Segmentation. [Online]. Available: Link

- Peng, Y. & Liu, R. (2010). Object Segmentation Based on Watershed and Graph Cut. CISP.

- Reliability Auditing Framework (n.d.). Reliability Auditing of Medical Image Segmentation under Signal Variability: The Catastrophic Failure Rate Framework. [Preprint].